Charley Lillard’s Twin Turbo 1969 Camaro

1021 Horsepower at 16 psi

The following components were used to complete the “new” Mule:

	Engine displacement – 389 cubic inches (4.135” bore x 3.625” stroke)

	GM aluminum race block

	Lunati 4340 crankshaft – 3.625” stroke

	JE Forged flat-top pistons (coated tops and skirts)

	Oliver 4340 billet steel connecting rods

	Clevite “H” series bearings

	Speed-Pro rings

	Cometic head gaskets

	Cam Motion custom grind solid roller camshaft

	Melling HV oil pump

	Fel-Pro gaskets

	ARP studs and bolts throughout engine

	MSD crank trigger

	18 degree TFS aluminum cylinder heads – 2.150” titanium intake/ 1.600” exhaust

	Isky valve springs and retainers

	ATI dampner

	Kinsler independent intake runners

	Kinsler fuel regulator

	F.A.S.T. 65 lb per hour fuel injectors

	W2W custom fabricated dual intake plenums

	W2W custom billet throttle bodies

	W2W custom exhaust headers – 1 7/8” 321 stainless

	Precision Turbo 63mm ball bearing turbochargers

	W2W custom fabricated air to air intercooler system

	W2W custom stainless heat shields throughout engine compartment

	Matching power steering reservoir and overflow tank

	Lightweight Startlite hoses with AN fittings throughout vehicle

	Vintage Air air conditioning system

	W2W fabricated core support bar with hood pins

	F.A.S.T. engine management system

	Reworked front suspension and rack and pinion

	Aeromotive high pressure fuel pump